# VNet-to-VNet VPN Implementation Using Azure VPN Gateway

Name: Ekta Mall

# Objective

To create a secure connection between two Azure Virtual Networks using VPN Gateway and verify connectivity using virtual machines.

# Detailed Implementation Steps

Step 1: Create Resource Group

Name: rg-vnet-vpn-lab
Region: Central India

Step 2: Create VNet1

Name: vnet1
Address space: 10.0.0.0/16
Subnet1: 10.0.1.0/24
GatewaySubnet: 10.0.255.0/27

Step 3: Create VNet2

Name: vnet2
Address space: 10.1.0.0/16
Subnet1: 10.1.1.0/24
GatewaySubnet: 10.1.255.0/27

Step 4: Create VPN Gateway for VNet1

Name: vnet1-gateway
Type: VPN
VPN Type: Route-based
SKU: VpnGw1
Public IP: pip-vnet1-gw
Active-active: Disabled
BGP: Disabled

Step 5: Create VPN Gateway for VNet2

Name: vnet2-gateway
Public IP: pip-vnet2-gw
Same configuration as above

Step 6: Create Connection

Connection Name: vnet1-to-vnet2
Type: VNet-to-VNet
Shared Key: Azure123

Step 7: Reverse Connection

Connection Name: vnet2-to-vnet1
Shared Key: Azure123

Step 8: Create Virtual Machines

VM1:
Name: vm1
VNet: vnet1
Subnet: subnet1
Public IP: Enabled
Username: azureuser
Password: (user-defined)

VM2:
Name: vm2
VNet: vnet2
Subnet: subnet1
Public IP: None (due to free subscription limit)
Username: azureuser1
Password: (user-defined)

Note: VM2 was created without public IP because Azure free subscription allows limited public IPs.

Step 9: Connectivity Test

From VM1:
ping 10.1.1.4
From VM2:
ping 10.0.1.4

# Conclusion

The VNet-to-VNet VPN was successfully created and verified using ping between virtual machines.